Attachment 1

STATE OF CALIFORNIA

Department of Fish Wildlife

Specifications for Interpretive Panels and Posts

- Definition

49 panels: dimensions 48"w x 54"h x 1/2"t;  high-pressure laminate on phenolic resin compounds OR gel coat laminate; UV, graffiti, and vandalism resistant, with 8 threaded inserts for 1/4" screws, ten year warranty

49 pairs (98 total) posts: 3"x 3" x 108" black powder coated aluminum; four holes in each post, 8 mounting brackets for each pair of posts

The panels will all be reprints of the same preexisting design.

- Delivery

To be shipped to:

Dept. of Fish and Wildlife

ATTN: Shannon Stanfield

1701 Nimbus Road, Suite A

Rancho Cordova, CA 95670